I2C Controller and Endpoint Device Model

Design Document - C Implementation

This document explains the simplified transaction-level I2C subsystem model. This describes the design, build the project, run the tests, and discuss future extensions.

# 1. Objective

This Implementation is a simplified I2C subsystem model with an MMIO-exposed I2C controller/master, one or more endpoint/slave devices, and a bus abstraction that routes transactions to the matching slave address.

The model is functional and transaction-level. It intentionally does not model SDA/SCL electrical waveforms, clock stretching, arbitration, or cycle-accurate timing.

# 2. Requirements Mapping

| Requirement | Implementation |
| --- | --- |
| MMIO controller registers | CONTROL, STATUS, TXDATA, RXDATA, and CMD are exposed as 32-bit MMIO words. |
| Commands | START, STOP, WRITE, and READ are supported through the CMD register. |
| Addressing | START consumes TXDATA as address byte: TXDATA[7:1] is 7-bit address; TXDATA[0] is R/W direction. |
| Read/write transactions | WRITE sends TXDATA to the endpoint. READ fetches one byte from endpoint into RXDATA. |
| Status reporting | BUSY, DONE, ERROR, and RX_VALID bits are implemented. |
| State machine | Controller maintains IDLE, START, SEND_ADDR, TRANSFER, STOP, and ERROR states. |
| Endpoint device address | Each endpoint has a fixed 7-bit address and responds only to that address. |
| Endpoint register map | Endpoint implements a 256-byte register map with Device ID, Status, and Data registers. |
| Bus abstraction | I2CBus stores endpoint pointers and routes lookups by 7-bit address. |

# 3. High-Level Architecture

+------------------+       MMIO read/write       +----------------------+
| Firmware / CPU   | --------------------------> | I2C Controller       |
| Bare-metal code  |                             | CONTROL/STATUS/etc.  |
+------------------+ <-------------------------- +----------+-----------+
        ^                                                |
        |                                                | transaction calls
        |                                                v
        |                                      +----------------------+
        |                                      | I2C Bus              |
        |                                      | address routing      |
        |                                      +----+------------+----+
        |                                           |            |
        |                                           v            v
        |                                  +-------------+  +-------------+
        |                                  | Endpoint    |  | Endpoint    |
        |                                  | addr 0x50   |  | addr 0x68   |
        |                                  | regs 0..255 |  | regs 0..255 |
        |                                  +-------------+  +-------------+

# 4. Source Layout

| File | Purpose |
| --- | --- |
| include/i2c_model.h | Public types, register offsets, bit definitions, return codes, and APIs. |
| src/i2c_model.c | Controller, bus, endpoint, command handling, state updates, and MMIO read/write implementation. |
| src/demo.c | Simple example program that creates endpoints, reads/writes registers, and shows error behavior. |
| tests/test_i2c_model.c | Unit-style tests for normal transactions and error paths. |
| Makefile | Build targets for test, demo, and clean. |
| README.md | Quick build/run guide and concise design overview. |

# 5. Controller Register Interface

| Offset | Register | Access | Description |
| --- | --- | --- | --- |
| 0x00 | CONTROL | R/W | Enable/configuration register. |
| 0x04 | STATUS | R/W1C | Busy, done, error, and RX valid status. |
| 0x08 | TXDATA | R/W | Address byte or transmit data byte. Only low 8 bits are used. |
| 0x0C | RXDATA | R | Received data byte. Only low 8 bits are used. |
| 0x10 | CMD | W | Command register. Read returns zero in this model. |

## 5.1 CONTROL Bits

| Bit | Name | Meaning |
| --- | --- | --- |
| 0 | ENABLE | Enables the I2C controller. |

## 5.2 STATUS Bits

| Bit | Name | Meaning |
| --- | --- | --- |
| 0 | BUSY | A transaction is active between START and STOP. |
| 1 | DONE | Last command completed. |
| 2 | ERROR | An error occurred. |
| 3 | RX_VALID | RXDATA contains valid read data. |

DONE, ERROR, and RX_VALID are sticky software-visible bits. Software can clear them by writing 1 to the corresponding STATUS bit. BUSY is live and cannot be cleared directly by software.

## 5.3 CMD Values

| Value | Command | Description |
| --- | --- | --- |
| 1 | START | Begin transaction or repeated START. |
| 2 | STOP | End active transaction. |
| 3 | WRITE | Send one byte from TXDATA to the selected endpoint. |
| 4 | READ | Read one byte from selected endpoint into RXDATA. |

# 6. Addressing Convention

During START, TXDATA is interpreted as an I2C address byte:

TXDATA[7:1] = 7-bit slave address
TXDATA[0]   = direction bit: 0 = write, 1 = read

Example:
Slave address 0x50, write phase: TXDATA = 0xA0
Slave address 0x50, read phase:  TXDATA = 0xA1

# 7. Controller State Machine

+------+    START     +-----------+   address match   +----------+
| IDLE | -----------> | START     | ----------------> | TRANSFER |
+------+              +-----------+                   +-----+----+
   ^                                                       |
   |                                                       | READ/WRITE
   |                                                       v
   |                                                  +----------+
   |                         STOP                     | TRANSFER |
   +--------------------------------------------------+----------+

Invalid command / missing slave / disabled controller / invalid STOP
        |
        v
+-------------+
| ERROR state |
+-------------+

| State | Description |
| --- | --- |
| IDLE | No active transaction. |
| START | START command has been accepted. |
| SEND_ADDR | Transient address-resolution step inside START. Not normally visible externally. |
| TRANSFER | Transaction is active and READ/WRITE commands are valid depending on direction. |
| STOP | Transient stop-processing step. |
| ERROR | An invalid sequence, disabled access, missing slave, or invalid command occurred. |

STOP is valid only when a transaction is active. STOP from IDLE or ERROR returns I2C_ERR_INVALID_SEQUENCE and sets the ERROR bit. This avoids masking firmware sequencing bugs.

# 8. Endpoint Device Model

Each endpoint has a fixed 7-bit address, a name, a device ID, a 256-byte register map, an internal register pointer, and small transaction state flags.

| Offset | Register | Description |
| --- | --- | --- |
| 0x00 | Device ID | Default initialized to the endpoint device_id. |
| 0x01 | Status | Generic endpoint status byte. |
| 0x02 | Data | Example data register used by tests. |

The endpoint follows a common register-style I2C convention: the first WRITE byte after address phase sets the internal register pointer. Subsequent WRITE bytes update registers starting at that pointer. READ returns the byte at the current pointer and auto-increments the pointer.

Simplification: register 0x00 / Device ID is writable in this model. This keeps the endpoint generic. A production model could add per-register access permissions to make it read-only.

# 9. Bus Abstraction

The I2CBus object stores up to I2C_MAX_ENDPOINTS endpoint pointers. It rejects duplicate slave addresses and routes START address resolution to the matching endpoint.

START command:
1. Extract 7-bit address from TXDATA[7:1]
2. Extract direction from TXDATA[0]
3. Call i2c_bus_find(bus, address)
4. If endpoint exists: begin transaction and enter TRANSFER
5. If endpoint does not exist: set ERROR and return I2C_ERR_NO_SLAVE

# 10. Example: Read Device ID from Slave 0x50

1. TXDATA = (0x50 << 1) | 0   // address + write
2. CMD    = START
3. TXDATA = 0x00                // internal register pointer = Device ID register
4. CMD    = WRITE
5. TXDATA = (0x50 << 1) | 1     // repeated START + read
6. CMD    = START
7. CMD    = READ
8. RXDATA now contains Device ID
9. CMD    = STOP

# 11. Error Handling

| Condition | Return Code | Visible STATUS |
| --- | --- | --- |
| Command issued while controller disabled | I2C_ERR_DISABLED | ERROR + DONE |
| START to missing slave address | I2C_ERR_NO_SLAVE | ERROR + DONE |
| READ without active read transaction | I2C_ERR_INVALID_SEQUENCE | ERROR + DONE |
| WRITE without active write transaction | I2C_ERR_INVALID_SEQUENCE | ERROR + DONE |
| STOP from IDLE or ERROR | I2C_ERR_INVALID_SEQUENCE | ERROR + DONE |
| Invalid CMD value | I2C_ERR_INVALID_COMMAND | ERROR + DONE |
| Duplicate endpoint address attach | I2C_ERR_DUPLICATE_ADDR | No controller status change |

The MMIO STATUS register exposes one generic ERROR bit. The C return code distinguishes no-slave/NACK-like failures from invalid command sequence. A future design could add an MMIO-visible ERROR_CAUSE register if firmware needs that distinction.

# 12. Design Notes from Review

| Observation | Resolution |
| --- | --- |
| Device ID writable | Documented as intentional simplification. Future extension can add register permissions. |
| SEND_ADDR transient | Documented. It is retained to show the natural state-machine step and future timing expansion path. |
| Address mask in i2c_bus_find | Simplified. Bus find now expects a 7-bit address; START passes the already-decoded address. |
| No NACK distinction in STATUS | Documented as future improvement. Function return codes already distinguish I2C_ERR_NO_SLAVE. |

# 13. Validation Strategy

The tests are written as firmware-like MMIO sequences. They verify both successful transactions and error paths.

| Test | Purpose |
| --- | --- |
| Device ID read | Verifies common register-read transaction using repeated START. |
| Register write/readback | Verifies endpoint register pointer and data update. |
| Multiple endpoints | Verifies bus routing by address. |
| Missing slave | Verifies no-slave error path. |
| READ without active transaction | Verifies invalid sequence handling. |
| STOP without active transaction | Verifies firmware sequencing bug detection. |
| Disabled controller command | Verifies CONTROL.ENABLE enforcement. |
| Duplicate address rejected | Verifies bus integrity. |
| STATUS write-one-to-clear | Verifies sticky status clearing semantics. |

# 14. Build and Run

make test
make demo
make clean

# 15. Assumptions and Limitations

Single I2C master only.

Multiple slave endpoints are supported.

7-bit addressing only.

Transaction-level modelling only; no SDA/SCL waveform simulation.

No clock stretching, arbitration, timing model, or multi-master behavior.

Repeated START is supported.

Endpoint register pointer convention is used for simple register reads/writes.

Device ID register is writable as an intentional simplification.

Generic ERROR status bit is used; detailed cause is available through C return codes.

# 16. Future Improvements

Add interrupt enable/status registers and an IRQ output line.

Add FIFO support for TX and RX.

Add per-register access permissions for endpoint registers.

Add an MMIO-visible error-cause register for NACK/invalid-sequence distinction.

Add command latency and event scheduling for timing-aware simulation.

Add trace logging hooks for debug